PRACTICE 1: INTRODUCTION TO LTSPICE

Full name:

Student ID:

Group:

Pre-practice: Install LTspice and add 035 library

1. Use LTspice software, take a NMOS and a PMOS and bias them such that |VGS| = 5V, |VDS| = 5V. Survey the drain current (Id), threshold voltage (Vth), and transconductance (gm) in cases:

- W=1um, L changes from 1um to 100um

- L=5um, W changes from 1um to 100um

Then, calculate uCox of MOSFETs. Compare NMOS and PMOS and then comment.

- NMOS with W=1um

| L (um) | Id (A) | Vth(V) | gm(S) | uCox |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 15 |  |  |  |  |
| 20 |  |  |  |  |
| 25 |  |  |  |  |
| 30 |  |  |  |  |
| 35 |  |  |  |  |
| 40 |  |  |  |  |
| 50 |  |  |  |  |
| 60 |  |  |  |  |
| 70 |  |  |  |  |
| 80 |  |  |  |  |
| 90 |  |  |  |  |
| 100 |  |  |  |  |

PMOS with W=1um

| L (um) | Id (A) | Vth(V) | gm(S) | uCox |
| --- | --- | --- | --- | --- |
| 1 | -1.940E-04 | -0.769 | 5.770E-05 | 1.364E-05 |
| 2 | -1.120E-04 | -0.776 | 3.660E-05 | 1.733E-05 |
| 3 | -7.910E-05 | -0.776 | 2.700E-05 | 1.918E-05 |
| 4 | -6.120E-05 | -0.775 | 2.140E-05 | 2.026E-05 |
| 5 | -5.000E-05 | -0.774 | 1.770E-05 | 2.094E-05 |
| 6 | -4.230E-05 | -0.773 | 1.510E-05 | 2.143E-05 |
| 7 | -3.660E-05 | -0.772 | 1.320E-05 | 2.185E-05 |
| 8 | -3.230E-05 | -0.772 | 1.170E-05 | 2.214E-05 |
| 9 | -2.890E-05 | -0.771 | 1.050E-05 | 2.235E-05 |
| 10 | -2.610E-05 | -0.771 | 9.560E-06 | 2.261E-05 |
| 15 | -1.770E-05 | -0.769 | 6.550E-06 | 2.322E-05 |
| 20 | -1.340E-05 | -0.768 | 4.980E-06 | 2.353E-05 |
| 25 | -1.080E-05 | -0.768 | 4.020E-06 | 2.375E-05 |
| 30 | -9.000E-06 | -0.768 | 3.370E-06 | 2.389E-05 |
| 35 | -7.730E-06 | -0.767 | 2.900E-06 | 2.398E-05 |
| 40 | -6.780E-06 | -0.767 | 2.550E-06 | 2.410E-05 |
| 50 | -5.430E-06 | -0.767 | 2.050E-06 | 2.421E-05 |
| 60 | -4.540E-06 | -0.767 | 1.710E-06 | 2.424E-05 |
| 70 | -3.890E-06 | -0.767 | 1.470E-06 | 2.431E-05 |
| 80 | -3.410E-06 | -0.767 | 1.290E-06 | 2.438E-05 |
| 90 | -3.030E-06 | -0.767 | 1.150E-06 | 2.445E-05 |
| 100 | -2.730E-06 | -0.766 | 1.030E-06 | 2.433E-05 |

- NMOS with L=5um

| W (um) | Id (A) | Vth(V) | gm(S) | uCox |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 15 |  |  |  |  |
| 20 |  |  |  |  |
| 25 |  |  |  |  |
| 30 |  |  |  |  |
| 35 |  |  |  |  |
| 40 |  |  |  |  |
| 50 |  |  |  |  |
| 60 |  |  |  |  |
| 70 |  |  |  |  |
| 80 |  |  |  |  |
| 90 |  |  |  |  |
| 100 |  |  |  |  |

PMOS with L=5um

| W (um) | Id (A) | Vth(V) | gm(S) | uCox |
| --- | --- | --- | --- | --- |
| 1 | -5.000E-05 | -0.774 | 1.770E-05 | 2.094E-05 |
| 2 | -1.080E-04 | -0.737 | 3.770E-05 | 2.211E-05 |
| 3 | -1.650E-04 | -0.724 | 5.770E-05 | 2.249E-05 |
| 4 | -2.230E-04 | -0.718 | 7.770E-05 | 2.268E-05 |
| 5 | -2.810E-04 | -0.715 | 9.770E-05 | 2.280E-05 |
| 6 | -3.390E-04 | -0.712 | 1.180E-04 | 2.293E-05 |
| 7 | -3.960E-04 | -0.711 | 1.380E-04 | 2.298E-05 |
| 8 | -4.540E-04 | -0.709 | 1.580E-04 | 2.301E-05 |
| 9 | -5.120E-04 | -0.708 | 1.780E-04 | 2.304E-05 |
| 10 | -5.700E-04 | -0.708 | 1.980E-04 | 2.307E-05 |
| 15 | -8.580E-04 | -0.705 | 2.980E-04 | 2.313E-05 |
| 20 | -1.150E-03 | -0.704 | 3.980E-04 | 2.316E-05 |
| 25 | -1.440E-03 | -0.703 | 4.980E-04 | 2.318E-05 |
| 30 | -1.720E-03 | -0.703 | 5.980E-04 | 2.319E-05 |
| 35 | -2.010E-03 | -0.703 | 6.980E-04 | 2.321E-05 |
| 40 | -2.300E-03 | -0.702 | 7.980E-04 | 2.321E-05 |
| 50 | -2.880E-03 | -0.702 | 9.980E-04 | 2.322E-05 |
| 60 | -3.460E-03 | -0.702 | 1.200E-03 | 2.327E-05 |
| 70 | -4.030E-03 | -0.702 | 1.400E-03 | 2.327E-05 |
| 80 | -4.610E-03 | -0.702 | 1.600E-03 | 2.327E-05 |
| 90 | -5.190E-03 | -0.701 | 1.800E-03 | 2.326E-05 |
| 100 | -5.770E-03 | -0.701 | 2.000E-03 | 2.326E-05 |

2. Plot the graphs of gm and μCox versus L. Comment?

3. Plot the graphs of gm and μCox versus W. Comment?